** ZMP Documentation **

1. Introduction.

ZMP is a communications/file transfer program for CP/M which

performs  Xmodem,  Xmodem-1k  (often erroneously called  Ymodem),

true Ymodem and Zmodem file transfer protocols.  Although  tested

with Z80DOS,  ZRDOS and CP/M 2.2, there seems to be no reason why

it shouldn't work with CP/M 3 as well.  The only requirements are

a  Z80 processor (sorry about that!),  a computer running CP/M in

one of its various guises, with at least 45k of TPA (but the more

the better!), and a modem.

When  you  try to pack this many features into one  program,

you end up with a pretty large file.  The big problem occurs when

file  transfers are attempted:  unless you have at least 4-8k  of

buffer size,  you might as well use xmodem protocol. The approach

taken in ZMP is to use overlays for various functions,  accepting

the  time  taken to load these from disk.  Thus performance  will

vary depending on your disk setup:  if you have a hard disk and a

fast processor you will likely not notice the difference.  If, on

the  other hand,  you are running a Commodore 128 with CP/M on  a

1571 drive,  there is no physical reason why ZMP won't work,  but

you might consider investing in a book to read while the overlays

load. (A suitable book to read might be a computer catalogue!).

The curious amongst you may notice that the beginning of the

ZMPX.COM  file has the magic 'Z3ENV' string,  but don't let  this

fool  you  into  thinking that you don't  need  to  add  terminal

characteristics  into the overlay if you have a ZCPR3 system.  It

has  proved  possible to persuade this particular C  compiler  to

access  ZCPR3's environment descriptor,  but not  for  ZMP.  Yet.

Perhaps later. In the meantime, the startup code is there for it.

In  order  to produce a program which would work  with  most

CP/M  systems,  the  Zmodem protocol performed by ZMP  is  fairly

simple.  The  transmit section uses 'Full Streaming with  Reverse

Interrupt',  as Chuck Forsberg calls it in his description of the

Zmodem protocol.  The receive section uses 'Segmented Streaming'.

This means that, if your system can do serial I/O and disk I/O at

the same time, ZMP does not take advantage of the faster transfer

rate  which this capability provides.  Since,  however,  I  can't

write and listen at the same time,  and neither can my  computer,

and  neither can the vast majority of CP/M computers,  it  seemed

the  best  approach to take.  Segmented Streaming means that  the

receive program tells the transmit program how big its buffer is.

The transmit program then sends just that much data,  then  waits

for  an acknowledge from the receiver.  We have encountered  some

Zmodem  programs  which send too much data in this  case:  errors

will appear if this happens,  but the protocol should recover and

the file will be received intact (we hope!).

The  string  which  ZMP passes to the receiving  program  to

interrupt  in  case of errors is likewise  simple.  Basically  it

causes  the  receiving program to send a control-C character  and

then wait for one second.  The receiver will then send its  ZRPOS

string, by which time ZMP, as the transmitter, should be ready to

receive it.

2. Customisation.

ZMP  must be customised to suit your system.  This  involves

overlaying  the  un-installed copy of ZMP.COM (contained in  this

library  as  ZMPX.COM) with a user-written installation  overlay.

Some  hints  on writing this are given below;  there is  a  blank

overlay  file in this library,  or you may be able to obtain  one

for  your  computer  from the same place you  got  this  library.

This value is set at 0145 hex, and should stay there permanently.

See  the notes in this document,  and also the ZMP-OVL.UPD  file,

for more details on how to set up your overlay.

Once the installation overlay is written,  assemble it  with

M80 (or SLR or whatever),  use RELHEX to create a .HEX file,  and

use  MLOAD  to overlay it over the ZMPX.COM file to produce  your

very own ZMP.COM.

3. Operation.

The following files must be on the same disk and user  area,

which must be specified in your customisation overlay:

ZMCONFIG.OVR -- the configuration overlay

ZMINIT.OVR   -- the initialisation overlay

ZMTERM.OVR   -- the terminal overlay

ZMXFER.OVR   -- the file transfer overlay

ZMP.HLP      -- the help file (recommended).

Start the program with ZMP.  The screen should clear, then a

title message is printed,  then ZMP enters terminal mode. You may

type  escape-H  for help at this point.  When you first  run  the

program,  the  first  thing  you  need to  do  is  to  enter  the

configuration  overlay  (type escape-C) and set all the  defaults

and  others to suit your system as required.  If you  don't  know

whether to change something or not, it's probably better to leave

it alone.  When you exit the configuration program, answer 'Y' to

the  'Make changes permanent' question.  ZMP.FON and ZMP.CFG will

be  produced  on your disk,  in the same drive/user area  as  the

above files.

Operation  of the program is controlled by escape  sequences

entered  in terminal mode.  Escape-H gives you a list of options.

Most of these are self-explanatory,  but they will be  summarized

here.

B - Send Break to modem.

If your overlay has been set up to send a break command

to your UART, this command will perform this function. (Some

remote systems may require a break sent to them to interrupt

Zmodem transfers).

C - Configure system.

This function is designed to set system defaults,  save

phone numbers etc. Changing baud rates etc. for a particular

call  are better handled with the escape-L  option.  If  you

answer Y or y to the 'make changes permanent?' question, the

new  configuration will be saved in a ZMP.CFG file,  and the

phone numbers in a ZMP.FON file for later use. The .CFG file

is read, if it exists, when ZMP is first started.

D - Get disk Directory.

Gets  a directory of the current drive and  user  area.

Change  to  another  with the  escape-F  command,  described

below.  The  directory  will  be  sorted  and  will  include

filesizes  unless  you  have so many files  on  the  current

drive/user area that there is insufficient memory  available

to  sort them.  In this case an unsorted directory,  without

file sizes, will be printed.

F - Disk/File operations.

This  command is used to change the current  drive/user

area,  to  reset a disk in the current drive,  and to  view,

erase, print or rename files. There are also options to give

a directory of the current drive/user area,  and to supply a

new filename for the capture file. This filename may specify

a different drive/user area than the current one. If capture

mode  is on,  the status line printed when terminal mode  is

entered will state the capture file name.

H - Get Help.

Prints the ZMP.HLP file. You may then either type CR to

return to terminal mode, or enter the required function key.

I - Initiate phone call.

Reads the ZMP.FON file, if any, and prints it. You have

four seconds after typing ESC I,  during which you may enter

the  letter corresponding to the required number,  in  which

case ZMP will dial it without printing a list. Otherwise the

full phone list will be printed, and you will be asked which

number you want.  Enter the identifying letter of the number

you  wish  to call.  You can also enter numbers not  in  the

list. Multiple numbers can be called by entering them separ-

ated  with  commas.  Dialling will then commence,  and  will

continue  until  one of the numbers answers,  or  until  you

abort  the process with the escape key.  Note that the  baud

rate  used  for the call is that in the .FON file  for  that

number,  or the current one if a new number is entered. This

function  works  best if the strings in the .CFG  file  have

been  set up for your modem,  and the initialisation  string

sent  to  the  modem sets it into verbose  mode  for  status

messages (ATV1).

K - Display Keyboard macros.

The configuration option allows up to ten macro keys to

be defined, and these are recalled by typing escape followed

by  the  numbers 0 to 9.  This function prints  the  current

assignments.

L - Change Line settings.

This  function allows temporary changes of  baud  rate,

stop bits, and data bits. There is also an option to operate

terminal  mode in full duplex (locally typed characters  are

sent but not displayed on the screen),  half duplex (locally

typed characters are sent and also displayed on the screen),

or  echo mode (as for half duplex,  but received  characters

are echoed to the remote system as well as being displayed).

Don't have two computers talking to each other in echo  mode

unless  you're bored.  There are also options to  allow/dis-

allow  control characters above CR to be displayed in  term-

inal mode,  to strip the parity bit in terminal mode, and to

re-initialise  the currently selected UART and modem at  the

current baud rate.

Like IMP,  ZMP uses location 003C hex to store a  value

corresponding to the present baud rate.  Then,  if you leave

ZMP  and  later re-enter,  it checks location  003C.  If  it

contains a legal value,  then that baud rate is set for you,

otherwise  it sets the default baud rate as selected in  the

.CFG file.

You  will  also find in this menu an option  to  enable

word  wrap in terminal mode,  and provision to switch to  an

alternate port. The word wrap function is rather simplistic,

but it does work. It functions only with characters typed at

the keyboard, and is designed to wrap on 80-character lines.

If  you are talking to a BBS which also has automatic  word-

wrap, it's probably best to leave this turned off, otherwise

you  could end up with double-spaced lines.  But if you  are

talking  to a human operator,  or if you are using a  packet

radio TNC, turn it on.

The alternate port switch allows use of two ports  with

ZMP. The status line (printed when terminal mode is entered)

will  display which port is in use.  The configuration  menu

allows  you to define which of the two is the default  port,

selected when ZMP is started up.

M - Toggle capture mode in Memory.

Received characters will be saved in a buffer and saved

in a file named 'ZMP.LOG' when the buffer fills or when  the

command is entered again.  A control-S/control-Q sequence is

sent  to the remote computer while the buffer is being saved

in an attempt to stop it sending more data.

P - Toggle Printer.

This  is similar to the 'M'  command,  except  incoming

characters  are sent to the printer.  This functions best if

your  system  performs  the  BIOS  'List  Status'   function

correctly.

Q - Quit the program.

Obvious.  You  will be asked if this is really what you

want.  Any  entry other than N or n will exit to CP/M.

R - Receive a file.

You  will be asked which protocol you wish to use.  The

default  is  ZMODEM.  The <X>modem option will allow  either

128-byte blocks (standard XMODEM) or 1k blocks  (XMODEM-1k),

since this is decided by the transmit end.  If an attempt is

made  to receive a file which has the same name as a file on

the current drive/user area, the current one will be renamed

to  .BAK  and the new one will then  be  received  normally.

(However,  see  below for the transfer resumption feature in

ZMP v1.5 and above).

Note that the byte count on the screen is not kept  up-

to-date on Zmodem receive.  This is because the data arrives

non-stop  and  there is simply no time available  with  non-

interrupt  driven computers to update the screen.  An update

is performed if errors occur,  and when the computer  pauses

to write to the disk.

Starting  with version 1.4,  Zmodem file  receive  will

commence  automatically upon receipt of the sender's ZRQINIT

string.  Thus all you need to do is have the sender initiate

the  transfer,  select  the  drive and user  area  you  wish

the file(s) to be received on, and wait..

ZMP  v1.5  adds the ability to  resume  an  interrupted

Zmodem  transfer.  If a Zmodem receive attempt fails (either

because of manual cancellation or massive errors),  you will

be asked if you wish to save the portion of the file already

received. If not, it will be erased. If so, and a subsequent

Zmodem  receive attempt would result in a file of  the  same

name,  you are asked if you wish to resume the transfer.  If

you  do,  transfer  will  start  at the  end  of  the  file.

Otherwise  the  old file will be renamed to .BAK as  before.

Since this feature is a function of the receiver,  it should

work   with   any  Zmodem  implementation   which   conforms

reasonably closely to Chuck Forsberg's standard. It has been

tested  with ZMP and RZMP,  and I would like to hear of  any

programs  with which it doesn't work.  No attempt is made to

determine  if the files are the same up to the  commencement

point:  the Zmodem protocol provides two ways in which  this

may  be determined (file date and CRC),  but neither has yet

been implemented in ZMP.

S - Send a file.

Operation   is   similar  to  the   receive   function.

Additional   options  available  are  ASCII  send  and   the

capability  of  distinguishing  between  normal  Xmodem  and

Xmodem-1k.  In  Ymodem and Zmodem modes,  wildcard filenames

and  multiple  filenames  are  allowed.  Multiple  filenames

should be entered separated by spaces.  In all cases,  files

on different drives/user areas may be specified by supplying

a zcpr3-style du: prefix (e.g. C7:NEATPROG.WOW).

Byte  count information is displayed in Zmodem mode  on

transmit. This causes noticeable breaks between packets, but

it is felt that this is outweighed by the usefulness of  the

information.  ZMP's Zmodem mode is capable of CRC-32  opera-

tion, although CRC-16 mode is used if the receiver is incap-

able of CRC-32.  Some other terminal programs,  however,  do

strange  things  when faced with a receiving  program  which

claims  it  can do CRC-32 (we have encountered one  for  the

Amiga  which exhibits this problem).  If this  happens,  the

esc-L  menu and the configuration overlay have an option  to

disable CRC-32.

U - Execute user-defined overlay.

This  feature  is designed to allow execution of  user-

produced overlays,  and other protocols which won't fit into

the   normal  transfer  overlay.   These  must   be   called

ZMxxxxxx.OVR,  and must be linked to be loaded at a specific

address  which will probably vary with different versions of

ZMP.  For v1.5, this address is 03ABAh. I may produce a list

of  available functions in the main ZMP.COM file for use  by

these overlays if there is enough interest.

After  typing  ESC  U,  you will be  prompted  for  the

overlay name.  The overlay will then be executed as  normal.

An  example  of  a  user overlay  is  ZMYAPP.OVR,  which  is

described  further  below.  Other  possibilities  include  a

Kermit overlay,  but as this would be a lengthy project, and

as I have no use at this time for a Kermit overlay,  whether

or not it gets done depends on the level of interest  shown.

If you are interested in a ZMP Kermit overlay, or if you can

think   of   any  other  overlays  which   could/should   be

implemented, please let me know.

X - Hangup.

This  function causes the modem to disconnect from  the

phone line, by momentarily dropping DTR.

Y - Print screen.

Allows the current screen to be dumped to printer. Note

that  this must be supported in the overlay:  most terminals

are incapable of this function.  The standard overlay prints

'This function not supported.'.  If you can make it work  on

your system, good luck!

Z - Clear screen.

Allows  the  screen to be cleared.  Useful if it  fills

with rubbish.

4. Hints for overlay writers.

Most  of the code is quite straight forward and will present

no problem. The section most likely to cause trouble is that part

concerned  with setting the required baud rate  and  (especially)

the  code for setting the correct parity,  number of stop  bit(s)

and data bits.

We  suggest  that you first get the main part  of  the  code

going by putting a return in the init: section at the point where

it  says "put your code here" This will let you check the rest of

the  code.  If the opening screen looks 'neat and tidy' then  you

know that 1) the code is in the right place,  and 2) your  cursor

addressing is working. Check the I/O by talking to your modem and

then  by  exchanging a file with another zmodem user/board  after

you set the I/O chip up with another program such as IMP.

Now  get the baud-setting working and then finally the  bit-

bashing required to set the parity, etc.

5. Other information.

a) ZMP at higher baud rates.

When  I  first produced ZMP,  I was more interested  in

producing  a  universal Zmodem program than  anything  else.

Originally (several C compilers ago!) I had difficulty  get-

ting it to work even at 300 baud,  and so little thought was

given to accommodate higher transmission speeds.  In partic-

ular, there is a "designed-in" bug/feature which would prob-

ably preclude ZMP working at much over 4800 baud.  The prob-

lem is in the user overlay,  in the mrd:  routine. The requ-

irement here is to have a routine which returns either  when

a  character  is  available at the modem (in which  case  we

return TRUE), or 100 mS has elapsed (in which case we return

FALSE).  The catch is that I used 100 x 1 mS waits,  between

which  we test for a character.  A little  calculation  will

show that a 9600 baud character will take a little over 1 mS

to transfer,  and 19200 baud characters take half this time.

Thus  we  are practically guaranteed to miss  characters  at

19200  baud,  and  even  9600 baud characters  leave  little

processing timeto spare.  Two possible ways to overcome this

are:

i)   Make the wait time shorter.  Thus we could wait 1000  x

100 uS periods instead. This, however, makes the actual

wait   time   more  unpredictable,   since   subroutine

call/return times are comparable to the wait  time.  It

also just puts off the evil day.

ii)  Use  a  hardware timer to determine whether 100 mS  has

elapsed.  This is the preferred approach. Thus the mrd:

routine  would loop continuously,  exiting when  either

there was a character at the modem or when the hardware

timer expired.  An embryo CP/M-68K version of ZMP using

this approach has proved capable of reliable  transfers

at  19,200  baud  (although one must admit that  it  IS

running a 68010 at 10 MHz!).

I  would  like  to hear from anyone  who  has  had  any

success with either of these two approaches.

b) The ZMYAPP overlay.

This  performs the amateur packet radio YAPP  protocol,

and  is accessed by the ESC-U function.  Operation is pretty

well  obvious,  but there are a couple of problems with  it,

depending on the hardware capabilities of your computer. The

main one is that, if the file to be received is greater than

the buffer size in ZMP,  the ZMP computer MUST be capable of

hardware handshaking (CTS/RTS). There appears to be no other

way  of stopping a TNC (which must be in  transparent  mode)

while we write the buffer to disk. The same thing will occur

if  the  TNC buffer fills:  if the TNC cannot stop  the  ZMP

program with CTS (DTR???) then its buffer will overrun.

The ZMYAPP overlay works fine with small files.  Again,

I would like to hear from anyone who has success with larger

files on different machines.

6. Acknowledgements.

ZMP was developed from Hal Maney's Heath-specific HMODEM II.

I  would like to thank Hal for writing HMODEM:  CP/M  users  have

been without ZMODEM capability for far too long.  As requested in

the  source  file,  acknowledgement  is  given  to  him  therein.

Appreciations  also go to the authors of the Hi-Tech C  compiler,

which proved to be capable of producing fast and compact code for

Z80 machines.

ZMP  in  its various incarnations is refined by  suggestions

from you,  the user.  In particular,  I would like to thank  Mike

Allen,  Richard  Kopplin  and  Fred Haines for  their  invaluable

suggestions.  I may sometimes be a little slow at implementation,

but I usually get there eventually!  Fred Haines has also  kindly

offered  to  be  the  U.S.  collection  point  for  bug  reports,

suggestions  etc.  His  address  appears at the  bottom  of  this

document,  and  he'll forward them to me via what he calls  'U.S.

Snail'.  I  will  try  and respond using what I  call  'Australia

Pest'.

I  would also like to thank Lindsay Allen,  sysop of  Z-Node

62.  His name was removed from the original zmp11 title screen at

his  own request,  since I had done most of the work in modifying

Hmodem to work on other machines. Without Lindsay's encouragement

at   difficult  times,   suggestions  as  to  how  to  go   about

recalcitrant procedures,  and experience in file  transfers,  ZMP

would not have been produced. Thank you.

7. Finally...

The files contained in this library are placed in the public

domain.  Just don't sell it,  claim you wrote it,  or do anything

similar that might annoy me.  Above all,  don't bother trying  to

sue  me  if it doesn't work,  or you tripped over  the  disk,  or

anything  similar.  I haven't distributed the source files partly

due to the size of them,  partly due to the fact that compilation

is messy (several modifications were needed to "standard" library

functions!),  and partly due to the fact that there's still  work

to do on them. Besides, I feel a certain fatherly feeling towards

ZMP,  having spent most of my spare time for the last four months

working on it. So here's the deal: I will continue to support ZMP

(and  RZMP)  until I get sick of it.  This could take an  unknown

amount of time!  At that point,  I will release the sources  into

the RCP/M community, and you may make of them what you will.

ZMP has a remote system relative,  called RZMP.  This allows

Zmodem  transfers  to  and from  remote  systems.  It  should  be

available from the same place from which you obtained ZMP.

I  have also produced an extremely cut-down version  of  ZMP

which  runs  under CP/M-68K,  currently running quite well  as  a

single  .68K  file (no overlays!) on a system with 128k bytes  of

memory.  If  there  is enough interest from CP/M-68k  users  (are

there any??), I could be persuaded to upgrade this version to the

point where it could be released.

Comments and suggestions are welcome. Bug reports are not so

welcome, but we'd like them anyway! Send either to:

Z-Node 62

Perth, Western Australia

(061+) 09-450-0200

(Soon to be on FidoNet)

U.S. users may send reports/comments to:

Fred Haines,

733 North King's Road, Apt. 356

Los Angeles, California 90069

-- Ron Murray

26th March, 1989

###############################################################